ACTIVE FILTER

Aim

To design and set up the active First order filters using Op-amp.

Apparatus Required

Op-amp, resistors, capacitors, breadboard, CRO, function generator and power supplies.

Design:

Fc=12ΠRC

Circuit Diagram

First Order LPF

First Order HPF

### Procedure

- Assemble the first-order active  filter circuit on the breadboard.

- Connect IC 741 to a ±15 V DC supply.

- Connect the function generator to the input of the filter.

- Apply a sinusoidal input, for example, 1 V  Vm

- Initially set the input frequency well below the calculated cut-off frequency.

- Observe Vin  and Vout  on the CRO/DSO.

- Measure the output voltage at different input frequencies.

- Calculate the voltage gain IN Decibel.

- Increase the frequency gradually through and beyond the expected cut-off frequency.

- Record the output voltage at each frequency.

- Identify the frequency at which the gain becomes approximately −3 dB from the passband gain.

- Plot the frequency-response curve of gain (dB) versus frequency.

- Compare the experimentally obtained cut-off frequency with the theoretical value.

### Observation Table

| S. No. | Frequency (Hz) | Vin (V) | Vout(V) | Gain | Gain (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | 30Hz | 1V | 1V | 1 | 0 |
| 2 | 50Hz | 1V | 1V | 1 | 0 |
| 3 | 80Hz | 1V | 1V | 1 | 0 |
| 4 | 100Hz | 1V | 1V | 1 | 0 |
| 5 | 150Hz | 1V | 1V | 1 | 0 |
| 6 | 500Hz | 1V | 500mV | 500 | -6.62 |
| 7 | 1kHz | 1V | 400mV | 400 | -7.9 |
| 8 | 2kHz | 1V | 200mV | 200 | -13.9 |
| 9 | 3Hz | 1V | 100mV | 100 | -20 |

Result:

Thus Designed the First order filters using op amp and obtained the results as Cutoff frequency of low pass filter is .........320Hz............